19 December 2017

# BUDGETARY COSTS FOR THE UPGRADING OF OMAN BLENDING SERVICES

Following on from the work undertaken to evaluate Oman Blending Services as a potential target for purchase, by Tasneea Oil and Gas Technology LLC, I was requested to revisit the budgetary pricing. Below is a new costing to an accuracy of +/- 15%. At this stage it is difficult to estimate to a smaller variance. This pricing is based on the following criteria –

## Construction estimates are based on prices from a local Omani EPC company (Precision World National LLC). Use of a different company may give higher figures. (PWN were the original company used for OBS)

## Pricing for the civil items are +/- 10%

## The design assumed is for a plant capable of a manufacturing a range of products requiring both heating and cooling and being capable of operating under pressures up to 50 psi.

## It is based on two independent reactors plus associated vessels. During a more detailed study at a later date this number may be revised.

## At the moment no specific chemical has been identified for manufacture but a range of potentially useful ones have been considered.

# CIVIL COSTINGS

These prices are calculated assuming that the adjacent plot in Samail is available and included in any purchase and the existing warehouses and tank farms are capable of handling the volume. As such the only building required is a manufacturing shed. This will house the dedicated manufacturing unit for production of key intermediates. These will then be blended to give the final products in the existing mixing vessels. Services will be extended from the existing site to provide power, nitrogen, compressed air, vapour abatement and sewage discharge.

The following prices are to an accuracy of +/- 10%

| Item | Cost in RO |
| --- | --- |
| Site levelling | 80,250 |
| Perimeter wall | 48,400 |
| Membrane underneath the surface to protect against chemical leaks | 8,000 |
| New production building | 55,000 |
| Concrete foundations | 35,000 |
| Internal roadways | 70,000 |
| Extension of services to new plot including sewage pipes | 40,000 |
| TOTAL | 336,650 |

# EQUIPMENT COSTINGS

It is assumed that two independent reactors are required. This will give flexibility and allow the production of smaller trial batches when necessary. The reactors will be heated and cooled using a thermal fluid and associated cooling system. This will avoid the need to install a steam boiler. In addition to the reactors a variety of vessels will be needed for the introduction of chemicals at a controlled rate and for the collection of condensate should the process require distillation. Final sizes for all these vessels including the reactors would be carried out in the next stage of the project.

It is assumed that all items required are manufactured to internationally recognised quality and safety standards. For the time being no particular supplier is suggested. This would happen during the next stage of the project.

The following prices are to an accuracy of +/- 15%

| Item | Cost in RO |
| --- | --- |
| Pressure vessel of 12,000 litre capacity | 80,000 |
| Pressure vessel of 6,000 litre capacity | 55,000 |
| Thermal fluid system | 125,000 |
| Cooling system | 65,000 |
| Heat exchangers | 60,000 |
| Associated feed and distillation vessels | 90,000 |
| Connecting pipework, pumps, instruments including load cells and their installation | 150,000 |
| TOTAL | 625,000 |

It should be remembered at this stage these pricings are estimated based on the information available today. If it is decided to move forward with the project it will require a significant amount of work to develop these ideas into a full package. Once it is clear what the plant is expected to deliver the pricing will be calculated to a more accurate total and if done in conjunction with a local EPC company would result in a fixed price lump sum amount.

I look forward to further discussions.

R. Plaxton BSc CSci CChem FRSC