|  | CATEGORY |  |  | PRODUCT CODE |
| --- | --- | --- | --- | --- |
|  |  |  |  | - Formulated methyldiethanolamine (MDEA) - Alkanolamine mixture - Alkanolamine mixture - Amine mixture - Formulated amines: 2,2’-(methylimino)diethanol; N-ethyldiethanolamine; piperazine - - Formulated amines: 2,2'-(methylimino)diethanol; N-ethyldiethanolamine; piperazine |
|  |  |  |  | - Nonylphenol polyethylene glycol ether (EO mole = 4) - Nonylphenol polyethylene glycol ether (EO mole = 8) - Nonylphenol polytheylene glycol ether (EO mole = 9) - Nonylphenol polytheylene glycol ether (EO mole = 10) - Ethoxylated primary alcohol (EO mole = 3) - Ethoxylated primary alcohol (EO mole = 5) - Ethoxylated primary alcohol (EO mole = 7) |
|  | WAX |  |  | - Polyethylene glycol (EO mole = 6) - Polyethylene glycol (EO mole = 12) - Polyethylene glycol (EO mole = 20) |
|  | COMMODITY PRODUCT |  |  | - Butyl diglycol ether - Butyl glycol ether - Butoxytriglycol - MEG - Mono Ethylene Glycol - Ethanolamines - DEA - Diethanolamine - TEA - Triethanolamine - Methanol - iso-Butanol |

| CATEGORY |  | PRODUCT CODE |  |  |  | DESCRIPTION / PURPOSE |
| --- | --- | --- | --- | --- | --- | --- |
| DEGREASER |  | (Quick Dry) |  |  |  | a solvent-based general purpose-cleaning compound, formulated with a unique blend of surfactants and organic cleaners. This unique blend of chemistries makes the removal of lubricant deposits easy and suitable for lubricant plant. Excellent for cleaning bi-metal lugs, busbars, fuse holders and most bare metal cores or parts safely to enhance their efficiency and conductivity. This product is also a non-flammable liquid, environmentally friendly and safer to handle it. |
|  |  |  |  |  |  | an effective and safe All- purpose Cleaner & Degreaser for use on any surface compatible with water. Its contains and uses the hyperactive colloidal particles (micelles) to penetrate and dislodge fat, oil, grease, protein and sugar from sur- faces. simply work by causing organic substance containing hydrocarbon molecules to repel from the surface, be suspended within the cleaning solution and easily removed by rinsing with water. |
|  |  | (Multipurpose Cleaner Heavy Duty) |  |  |  | oil and grease stains removal cleaner, formulated for efficient oil plat- form cleaning with low foaming characteristic. It consists of biodegradable non-ionic surfactants to deal with various extent of cleaning requirement range from light to heavy duty. The product shows good degreasing performance on hard metal surface without causing any corrosion and damaging to the applied surface due to the mild formulation. Using high pressure washer, the rig wash capable to clean small and narrow areas where brusher hardly each to sweep and wipe away the contaminated stains. |
|  |  | (Multipurpose Cleaner Light Duty) |  |  |  | an aqueous-based detergent consists of proprietary blend of non- ionic surfactants which is readily biodegradable. It provides good degreasing performance on hard surface and develops low foaming during washing that benefits cost and energy saving in rinsing step. Due to its low surface tension characteristics, this product capable to provide good wetting and degreasing properties without causing any corrosion and damaging to applied surface. |
|  |  |  |  |  |  | an aqueous-based detergent consists of proprietary blend of non- ionic surfactants which is readily biodegradable. It provides good degreasing performance on hard surface and develops low foaming during washing that benefits cost and energy saving in rinsing step. Due to its low surface tension characteristics, this product capable to provide good wetting and degreasing properties without causing any corrosion and damaging to applied surface. |
|  |  |  |  |  |  | a green based, biodegradable industrial and institutional (I&I) multi- purpose cleaner, targeted on cleaning of light to heavy oil contamination on metal surface. Emulsion based characteristics of the product clean oily soiled surface ther modynamically with minimum work required thus help to save energy and time consumption for additional mechanical washing such as scrubbing, brushing and wiping. |
|  |  | (Hand Wash) |  |  |  | an effective solvent free soap with unique ecologic formulation. Extremely efficient and soft for hand. It clears all kind of dirt on hands. Gentle to the human skin yet effectively removes grease and other tough stains. It contains and uses the hyperactive colloidal particles (micelles) to penetrate and dislodge fat, oil, grease, protein and sugar from surfaces. simply work by causing organic substance containing hydrocarbon molecules to repel from the surface be sus- pended within the cleaning solution and easily removed by rinsing with water. |

| CATEGORY |  | PRODUCT CODE |  |  | DESCRIPTION / PURPOSE |
| --- | --- | --- | --- | --- | --- |
| DEFOAMER |  |  |  |  | polymer of propylene glycol. Chemically it is a polyether. The term polypropylene glycol is reserved for low to medium range molar mass poly mer when the nature of the end-group, which is usually a hydroxyl group, still matters. |
|  |  |  |  |  | low foaming surfactant that will lower the surface tension of foams, thus causing the foam bubbles to break up and those beneath the surface to grow to the point where they are large enough to float to the surface and break. It suppresses foam during preparing drilling fluid with additives in all types of water based mud including near salt saturated water. |
|  |  |  |  |  | mixture of non ionic surfactant which forming an antifoam product which has been developed for use in the oil and gas industry to solve foaming problems in the following systems: - Sea water deaeration towers - Water systems in general |
|  |  |  |  |  | solution of highly concentrated sulfonate salt used effective in aqueous or law water applications and also in oil and gas separation. The product accelerates the release of dissolve gases, reduces foam carryover and increases separation efficiency. |
| DESCALER |  |  |  |  | blend of non-hazardous, non-corrosive, biodegradable descaler. It is designed to remove carbonate (CO3) based deposits. It also has the ability to dissolve and remove deposits caused by corrosion byproducts (e.g.: Iron Carbonate, Zinc Carbonated). It is designed to dissolve carbonate deposits found in all types of water operated equipment such as boilers, cooling towers including galvanized towers, closed loops, heat exchangers and aluminum molds. |
|  |  |  |  |  | blend of an aqueous salt solution and as such, it should not be considered hazardous when used under normal conditions. Laboratory testing shows this material to be biodegradable. This biodegradable product is very efficient in re- moving a scale, rust and marine growth from heat exchangers, boilers, outboard motors, heating systems, car engine maintenance and etc. |
|  |  |  |  |  | blend of an aqueous salt solution and as such, it should not be considered hazardous when used under normal conditions. Laboratory testing shows this material to be biodegradable. This biodegradable product is very efficient in re- moving a scale, rust and marine growth from heat exchangers, boilers, outboard motors, heating systems, car engine maintenance and etc. |

| CATEGORY |  | PRODUCT CODE |  |  | DESCRIPTION / PURPOSE |
| --- | --- | --- | --- | --- | --- |
| SCALE INHIBITOR |  |  |  |  | a specialized blend of phosphonate based water soluble scale inhibitors suitable for high temperature and high pressure environment. It is an excellent Calcium Carbonate, Calcium Sulfate and even Barium and Strontium Sul- fate inhibitor suitable for applications where the mentioned mineral scales are a problem. The product is an excellent threshold inhibitor with added sequestering function, making it suitable also for Iron deposit inhibition. |
|  |  |  |  |  | concentrated water and brine soluble organic phosphonate developed for formulating scale inhibitors for use in water flood operations, water disposal systems and producing oil wells. Formulations with the product are highly effective in inhibiting both Calcium Carbonate and Sulfate scale formation by interruption of the crystal growth process. |
| CORROSION INHIBITOR |  |  |  |  | a water-soluble, oil dispersible corrosion inhibitor that offers excel- lent protection from both CO2 and H2S corrosion. It is effective in oil production conditions. |
|  |  |  |  |  | a blend on an Imidazoline salt that produces a very effective and stable film on downhole and process surfaces, together with a novel Amine derivative, which improves the water solubility of the product. It is a water-soluble, highly oil dispersible organic corrosion inhibitor. It has been designed for use in pipelines and production equipment for the protection of oil and gas process systems. Being water soluble, the corrosion inhibitor will effectively protect water- treatment facilities downstream of production equipment. |
|  |  |  |  |  | oil-soluble, water-dispersible, film forming corrosion inhibitor for use in gas-production systems and is effective against both “sweet” and “sour” corrosion. The inhibitor is specifically designed for continuous injection in gas trans- mission lines, gas storage systems and especially where it is likely to come in contact with gas compression units. The product at high temperatures without gunking in the system. It promotes removal of fouling material in gas compressors, prevents additional fouling and inhibits the corrosion process. |
|  |  |  |  |  | oil-soluble, water-dispersible, film forming corrosion inhibitor for use in gas-production systems and is effective against both “sweet” and “sour” corrosion. inhibitor is specifically designed for continuous injection in gas trans- mission lines, gas storage systems and especially where it is likely to come in contact with gas compression units. The product at high temperatures without gunking in the system. It promotes removal of fouling material in gas compressors, prevents additional fouling and inhibits the corrosion process. |
|  |  |  |  |  | a corrosion inhibitor base developed especially to meet the needs of the oil and gas industry. it is a high quality based Imidazoline that has the potential to be formulated to meet a broad spectrum of inhibitor requirements. Its formulations will provide excellent protection in most treating enviro ments. |

| CATEGORY |  |  | PRODUCT CODE |  | DESCRIPTION / PURPOSE |
| --- | --- | --- | --- | --- | --- |
| CORROSION INHIBITOR |  |  |  |  | water-soluble, highly oil dispersible organic corrosion inhibitor. It is a blend of an Imidazoline salt with other surfactants. It has been formulated to prevent CO2 and H2S corrosion in pipelines and production equipment in oil and gas process systems. It has been specifically developed for high shear appli cations, typically found in gas systems. |
|  |  |  |  |  | oil-soluble, water-dispersible, film forming corrosion inhibitor for use in gas-production systems and is effective against both “sweet” and “sour” corrosion. inhibitor is specifically designed for continuous injection in gas transmission lines, gas storage systems and especially where it is likely to come in contact with gas compression units. It also provides excellent protection making it an ideal product for downhole treating. |
| DEMULSIFIER |  |  |  |  | demulsifiers which are very effective synergistic blends of high molecular weight oxyalkylated glycols, oxyalkylated resins and amines in heavy aromatic solvent. |
|  |  |  |  |  | effective synergistic blends of high molecular weight oxyalky- lated glycols, oxyalkylated resins and amines in heavy aromatic solvent. The actual formulations of. |
| DEOILER |  |  |  |  | dispersions are low to medium molecular weight, water soluble, cationic, acrylamide-based copolymers supplied as oil free dispersions of polymer in wa ter. They are effective over a pH range of 3 – 10. Its dispersions can be used to treat waste water produced by a wide variety of oil industry operations. It functions by bridging between oil droplets and/or suspended solids to form a large agglomerated mass or floc which can be easily removed. It can be effectively utilized in a broad range of liquid-solid separation processes. |
|  |  |  |  |  | a solution of an anionic polymer for use as a metal precipitating agent. It is effective over the pH range of 3 – 10. |
|  |  |  |  |  | a high molecular weight, acrylic emulsion copolymer for use as a flocculant in oilfield water clarification. The product is supplied in an acidic form and requires neutralization before use. Its functions by reducing the repulsive surface charges on oil droplets and certain organic and inorganic particulates found in produced oilfield waters. This helps promote the formation of a larger floc and improves settling or filtration rates. It is often used in conjunction with inorganic coagulants or cationic polyelectrolytes. |

| CATEGORY | PRODUCT CODE | DESCRIPTION / PURPOSE |
| --- | --- | --- |
| DEOILER |  | high molecular weight polymer with high charge polymer. They are applied to the wastewater containing great quantity of organic colloid such as the wastewater from dyeing, papermaking, and petroleum industries. |
|  |  | dispersions are low to medium molecular weight, water soluble, cationic, acrylamide-based copolymers supplied as oil free dispersions of polymer in wa ter. They are effective over a pH range of 3 – 10. Its dispersions can be used to treat waste water produced by a wide variety of oil industry operations. It functions by bridging between oil droplets and/or suspended solids to form a large agglomerated mass or floc which can be easily removed. It can be effectively utilized in a broad range of liquid – solid separation processes such as settling, thickening, dissolved air flotation and centrifugation. |
| BIOCIDE |  | water-soluble concentrated biocide for use in oil, water and gas production. It also recommended for used in open recirculating water system. it is glutraldehyde based compound and posses the following special prop erties: - Able to remove established bioflim and inhibit re-growth - Broad spectrum biocide kills aerobic and anaerobic microorganism, including sulphate-reducing bacteria - Kills algae and remove algae deposits - Effective in broad range of pH and temperature - Fast acting - Odorless and non-corrosive to metals at recommended use level - Chemically compatible with common corrosion and scale inhibitor and dispersant - Surface action facilitate penetration to microorganisms protected by biologically or chemical induced fouling - Water soluble, easy to mix and dilute uniformly |
|  |  | a new class of anti-microbacterial agent which offers superior anti- microbacterial activity controlling the growth of bacteria, algae and fungi, with mini- mum effects on the environment. THPS based compound. THPS’s benefits include low toxicity, low dosage and rapid breakdown in the environment. |
| POUR POINT DEPRESSANT |  | is the pour point depressant which especially developed for the treat ment of crude oils. It is used to lower the pour point by acting directly on the paraffinic crystal growth. In addition, its molecular structure provides excellent dispersing properties allowing the paraffinic microcrystal to remain in suspension. |
|  |  | is one of the pour point depressant which especially developed for the treatment of crude oils. It is used to lower the pour point by acting directly on the paraffinic crystal growth. In addition, its molecular structure provides excellent dispersing properties allowing the paraffinic microcrystal to remain in suspension. |

| CATEGORY | PRODUCT CODE | DESCRIPTION / PURPOSE |
| --- | --- | --- |
| DECONTAMINANT |  | a specifically formulated chemical that will eliminate toxic gases and hydrocarbon vapor during plant turnaround and storage tank decontamination. it is water soluble and contains alcohol ethoxylate based com- pound with special enzymes cultured in the complex detergent system. It is effective in removing contaminants produced by various systems in oil/gas pro- duction and petrochemical plants. It is extremely fast acting and works by converting hydrogen sulfide and hydro- carbon vapor into water-soluble compound with low Chemical Oxygen Demands (COD). |
|  |  | a formulated chelating agent, designated to transform elemental and compound mercury found in vapor into stable mercury salt in an aqueous form. It is a powerful solution that is able to capture mercury vapor from the atmosphere. it is formulated to function aggressively at both elevated and ambient temperatures. The product contains additives to ensure compatibility with various system, materials and various application methods. |
| SCAVANGER | (H2S Scavenger) | the H2S gas scavengers which remove hydrogen sulfide (H2S) from process systems to improve safety, protect equipment and meet sales specifications. For safe, effective removal of H2S in ranges from 1-to-200 pounds per day, it is available in a variety of formulations for complete and partial H2S removal in oil- and water-soluble applications. Where scavenger-induced calcium carbonate scaling is a problem, and it provides an effective scale inhibi tor. |
|  | (Oxygen Scavenger) | a solution of highly concentrated ammonium bisulfite used to effectively remove dissolved oxygen to trace level at low operating temperature. The product is highly effective in minimizing the effect of oxygen corrosion in various industrial applications with or without mechanical deaeration units. |
| FLOURESCENT DYE |  | specializes in the provision of subsea leak detection services for the offshore pipeline industry. It help to prevent serious environmental damage by promptly detecting the leakage of hydrocarbons - whether gas or oil - from risers, subsea control systems, water injection systems and other subsea facilities. This product is capable of detecting very small leaks and has been used successfully for a number of years by oil companies and offshore operators worldwide. |
| GAS ODORANT |  | a tert-butyl mercaptan based products which will be added in the gas distribution system. It offers a technical grade quality of gas odorant, far exceeding the commercial grade specifications. The odor from the product give a distinctive and unpleasant odor so that the presence of gas in air in concentrations below the lower explosive limit (LEL) is readily detectable. By adding the product into the gas system, it also will not changed or disturb any physical or chemical property (except smell) of the natural gas. Generally speaking, in the process of natural gas delivering for both public and industrial use. |